GCSE Science                Unit – P2

Topic: Electrical Current - Lesson 5

Activity: Components in parallel

Spec Refs: 11.4

Pages:  p37 - 38

Objectives (including specific refs to HSW):

- That the potential difference across components in parallel is same.

- That the total current in a parallel circuit is the sum of the currents in the individual branches.

Outcomes:

Most students should be able to:

- Recognise components in parallel with each other.

- Calculate the current in a branch of a parallel circuit, given the total current and the current in the other branches.

- Identify, for resistors of known resistance in parallel, which resistor has the most current passing through it and which has the least.

Some students should also be able to:

- Analyse simple parallel circuits to find the current through branches and the p.d. across components.

Suggested Activities (including specific refs to HSW):

Starter:

Ask students what the word parallel means and then give as many uses as possible.

Main:

- Show a circuit with bulbs in parallel ask student to discuss why they think they are brighter than bulbs in series.

- Students should then investigate current in a parallel circuit.  (Students find this practical difficult so the teacher needs to give step by step instruction, or it could be used as a demonstration). They should find out that current is shared.

- Students then should investigate p.d, in a parallel circuit, this is much easier to do. They should find out that the p.d. is the same.

- Students can then go back to the discussion on why bulbs in parallel are brighter now they have more information.

- Students could answer the questions from Collins p252 – 253

Plenary: Students complete ‘Parallel Circuits’ in workbook (HT p139 – 140/FT p124 - 125).

Apparatus / Materials Required

P2.22 ‘ Brighter bulbs’ card sort, Nelson Thornes textbook, AQA Collins textbook and student  workbook.

Class Practical – Power supplies, voltmeters, ammeters, bulb, wires, resistors.

Safety:

Make sure that the current is not set too high and that students don’t short the circuit.

References:

Nelson Thornes – pages 242 – 243

Collins – pages 252 - 253

Differentiation

Extension: Students could use an analogy to explain the difference between a series and parallel circuit.

Support: Provide the students with the circuit rules.

Homework ideas:

P2.21b    Current and resistance in circuits